LEGAL DESCRIPTION - SUBJECT PROPERTY (AS SURVEYED)

COMMENCING AT THE NORTHEAST CORNER OF SECTION 20, TOWNSHIP 5 SOUTH, RANGE 2 EAST, BALDWIN COUNTY, ALABAMA, THENCE N 89°32'26" W ALONG THE SECTION LINE 1064 FEET; THENCE S 00°20'31" W 40.00 FEET TO A FOUND 1" OPEN TOP ON THE SOUTH LINE OF DAPHNE AVENUE (80' RIGHT-OF-WAY) FOR THE POINT OF BEGINNING OF THE PROPERTY HEREIN DESCIRBED; THENCE S 03°01'32" W 216.53 FEET TO A FOUND 1/2" REBAR CAP, ILLEGIBLE; THENCE S 89°07'02" W 274.45 FEET FOUND 5/8" REBAR; THENCE N 01°04'58" E 222.10 FEET TO A SET 1/2" REBAR (PLS CA#1067-LS) ON THE SOUTH LINE OF DAPHNE AVENUE (80' RIGHT OF WAY); THENCE S 89°40'24" E ALONG SAID SOUTH LINE 281.66 FEET TO THE POINT OF BEGINNING, CONTAINING 1.40 ACRES MORE OR LESS.